USB 2.0 Single Step Transaction Debugger 
User’s Guide

- Version 1.0

1.0 Overview

2.0 Application Requirements

3.0 Running the Program

- 4.0 Menus and Fields

- 4.1 Main Property Sheet

4.1.1. Device/Transfer Information

4.1.2 Data Buffer

4.1.3. History

4.1.4. Other Buttons

- 4.2 Config Setup Packets Property Sheet

4.2.1 Clear Feature

4.2.2 Get Configuration

4.2.3 Get Descriptor

4.2.4 Get Interface

4.2.5 Get Status

4.2.6 Set Address

4.2.7 Set Configuration

4.2.8 Set Descriptor

4.2.9 Set Feature

4.2.10 Set Interface

- 5.0 Example - Executing a Control Transfer Transaction Using SSTD

6.0 Troubleshooting

- 1.0 Overview											Back to Top

The Universal Serial Bus 2.0 Single Step Transaction Debugger program (USB2SSTD.exe) enables USB 2.0 developers to interact with their prototype devices in a transaction-based manner. This application is a tool with which you can watch, debug, check and analyze device characteristics and compliance. The tool and this user guide provide all the information necessary to perform transaction-based interaction. The USB 2.0 Single Step Transaction Debugger is referred to as “SSTD” in this document.

- 2.0 Application Requirements								Back to Top

- 2.1 Hardware

SSTD requires the installation of one high-speed host controller into the host computer system. (SSTD testing was performed with the NEC-based host controller available in the USB-IF Product Development Kit.) The computer system must meet the minimum configuration required by the Microsoft Windows* 2000 Professional operating system (133 MHz performance level or better processor, 64 MB RAM installed), with a processor speed of 300 MHz performance level or better and 128 MB RAM installed strongly recommended. One high-speed hub or other device must be connected to the high-speed host controller. SSTD has been tested only with high-speed devices. It is strongly recommended that the device under test is the only device connected to the high-speed USB 2.0 bus.

- 2.2 Operating System

Currently, SSTD can execute only on the Microsoft Windows 2000 operating system.

- 2.3 Other Software

The pre-release USB 2.0 stack available from Microsoft or included in the USB-IF Product Development Kit must be installed.

- 3.0 Running the Program									Back to Top

There is no installation script for SSTD. The recommended way to install is to copy the program file from the installation medium or image and paste it to the desktop or any directory. If there is only one high-speed USB controller in the system, SSTD is ready to operate and displays the Main property sheet (see section 4.1). If you install two or more high-speed USB host controllers, a dialog box prompts you to select the host controller connected to the device under test. After selecting the appropriate host controller, the Main property sheet appears.

- 4.0 Menus and Fields  										Back to Top

- 4.1 Main Property Sheet

Figure 4.1 shows SSTD’s Main property sheet:

- Figure 4-1 Main property sheet

- 4.1.1. Device/Transfer Information								Back to Top

Speed (choice field)
Always set the speed field to High in this version of SSTD.

PID (choice field)
This drop-down box contains three options:

- SETUP

- IN

- OUT

For all standard device requests, SSTD toggles the PID correctly. For bulk and isochronous transfers, you must set PID to IN or OUT, according to the desired transfer direction.

When the application starts, SETUP is displayed. This maintains the last-used PID. If a standard device request is selected from the “select Transaction dialog box,” it then resets to SETUP. For Bulk transfer (IN/OUT), you must set the properties manually.

Data Toggle (choice field)
For all standard device requests, SSTD toggles the Data Toggle field correctly. For vendor and other types of requests, you must set this field to Data0 or Data1 (respectively), to use an even or odd data packet synchronization bit.

There are three types of requests:

- Standard device requests

- Class device requests as implemented by Microsoft

- Vendor device requests as implemented by IHVs

The default is Data0 at program startup.

Time Out (numeric field — decimal) 
You can specify the time SSTD waits for the indicated transaction to complete. The time entered is expressed in milliseconds. A value of zero (the default) indicates that you want to wait indefinitely for transaction completion.

Transfer Type (choice field)
Most types of transfers are Asynchronous (default). For constant rate, “real-time” transfers, set the field to Isochronous.

Device Address (numeric field — decimal)
You are responsible for entering the correct address here. If the device under test is the only device connected to the high-speed USB (highly recommended; see section 2.1), then the device address should be one (1). Please be sure to note that the device address appears as a DECIMAL number on the Main property sheet.

Endpoint Address (numeric field — decimal) 
The default value is zero (0) to indicate the default control pipe. To send requests to other endpoints in the device, type the desired endpoint number into this field.

Error Counter (choice field) 
This counter enables you to set the number of times the host controller should attempt to process the transaction before reporting an error. Valid values are zero (0), one (1), two (2) or the default three (3).

- 4.1.2 Data Buffer											Back to Top

Select Transaction (choice field) 
The following are SSTD’s supported standard device requests:

- Clear Feature

- Get Configuration

- Get Descriptor

- Get Interface

- Get Status

- Set Address

- Set Configuration

- Set Descriptor

- Set Feature

- Set Interface

You must select which device request to send to the device. After selecting the request, the device request appears in the transmit/receive buffer area for you to review before executing.

Clear Buffer (button)
Clicking this button clears the transmit/receive buffer.

Packet Size/ i.e Bytes To Transmit (numeric field - decimal) 
SSTD automatically manages this field for the standard USB device requests. If IN or OUT is selected in the PID field (for example, to start a bulk transfer), then you must enter the number of bytes he wants sent to or received from the device. When the transaction has successfully completed, the number of bytes displayed is the lesser of 1) your specified number of bytes or 2) the actual number of bytes sent to or received from the transmit/receive buffer.

- 4.1.3. History												Back to Top

Logfile (string field)
SSTD saves all attempted transactions between the host and device, as well as the transaction results, in the history buffer. By default, this log is in a file named “usb2sstd.log,” located in the same directory as the SSTD program file. Any user-desired file name entered into the Logfile field may be used, but there is currently no mechanism by which to change the default directory location.

View (button)
Clicking this button opens the Logfile in the Microsoft Windows Notepad* application.

Save (button)
Clicking this button saves the history buffer as a file, named as specified in the Logfile field.

Clear (button)
Clicking this button resets the history buffer. Note: You cannot recover history data unless you save it before clearing the buffer.

- 4.1.4. Other buttons											Back to Top

Enable port (button)
Clicking this button puts the specified hub port into test mode. (Please see the “Test Mode” field description in section 4.2.9.)

Connect  (button)
Clicking this button simulates a chirp, a handshake between the upstream port and a high-speed device, to identify that the device is high speed.

Execute (button)
Clicking this button executes the transaction specified in the transmit/receive buffer.

Exit (button)
Clicking this button exits the SSTD application. Please note that the application will NOT ask if you want to save the logfile before you exit the program; you must have previously used the Save button, as previously described.

- 4.2 Config Setup Packets Property Sheet							Back to Top

Figure 4-2 is an example of the Config Setup Packets property sheet:

- Figure 4-2 Config Setup Packets property sheet

SSTD maintains the parameters particular to a standard USB device request. All changeable parameters are listed, grouped and named with the corresponding device request. For example, to use the Set Address command request (9.4.6), the only required field is Device Address. To input a value for this parameter, click the tab for the Config Setup Packets property sheet, find the group of parameters named “Set Address”, and within that group change the value in the Device Address. You can elect to use the SSTD default Device Address, which is zero (0), by clicking the tab for the Main property sheet, selecting the ”Set Address” request from the “Select Transaction” choice field, and clicking the Execute button.

Details of each available command’s parameters follow.

4.2.1 Clear Feature											Back to Top
There are three fields for you to set.

Feature (choice field)  
Depending on the device request, this field has a maximum of three choices:

- Remote Wakeup (default if Device is selected as Recipient)

- Endpoint Halt (default if Endpoint is selected as Recipient)

- Test Mode

Recipient (choice field)
The recipient can be either a Device or an Endpoint. The default is Device, but modifying the Feature field automatically changes the Device.

Recipient Address (numeric field – hexadecimal)
If the recipient is an endpoint, you enter the desired endpoint address in this field; the default is zero (0x0). If the recipient is a device, the address is zero (0x0) and the field is ignored.

4.2.2 Get Configuration	 								Back to Top
There are no fields for you to set for this packet.

4.2.3 Get Descriptor		 								Back to Top

There are four fields for you to set.

Type (choice field)   
This drop-down box lists the all descriptor types. The initial default is Device, but the last selection is retained once you change the field.

- Device

- Configuration

- String

- Device Qualifier

- Other Speed Configuration

Index (numeric field — hexadecimal)
Enter the correct index value from the device configuration descriptor data into this field, if you select the String or Configuration descriptor type. The default is zero (0x0000), in order to retrieve the USB_Language_IDs supported by the device. (Note that if the device does not support any string descriptors, the value 0x0203 is returned.)

Language ID (numeric field — hexadecimal) 
Set this field to USB_Language_ID if the String descriptor type is selected. Entering the default ID zero (0x0000) returns the correct valid string. The code for English is 0x0009 and the subcode for U.S. English is 0x0004.

Length (numeric field – hexadecimal)
This is the number of bytes sent to or received from the device. If the length is known, then SSTD automatically changes to the correct value. For unknown lengths, the default value is 255 bytes (0xFF).

4.2.4 Get Interface	 									Back to Top
There is one field to input.

Interface (numeric field – hexadecimal)
Set this field to the Interface number to use. The default is zero (0x0).

4.2.5 Get Status		 								Back to Top
You must input the data in two fields.

Recipient (choice field)
The recipient can be either a Device or an Endpoint. The default is Device.

Recipient Address (numeric field – hexadecimal)
If the recipient is an endpoint, you should type the desired endpoint address in this field; the default is zero (0x0). If the recipient is a device, the address is zero (0x0) and the field is ignored.

4.2.6 Set Address		 								Back to Top
There is one field for you to specify.

Dev Addr (numeric field – hexadecimal) 
This field contains the desired device address value to use with “Set Address” command request. Please note that this field is interpreted as a hexadecimal value.

4.2.7 Set Configuration	 								Back to Top
You must specify one field.

Cfg Value (numeric field – hexadecimal) 
This is the desired configuration value to use with “Set Configuration” device request. The default is zero (0x0).

4.2.8 Set Descriptor		 								Back to Top
There are four values to specify; they are the same as for the “Get Descriptor” command (see section 4.2.3).

4.2.9 Set Feature		 								Back to Top
You must specify four values; the first three are the same as in the “Clear Feature” command (see section 4.2.1 above). The fourth value to be input is shown below.

Test Mode (choice field)
This field lists all of the standard device test modes, allowing you to select the appropriate one.

- Test_J

- Test_K

- Test_SE0_NAK

- Test_Packet

- Test_Force_Enable

- N/A (default)

4.2.10 Set Interface		 								Back to Top
There are two values for you to input.

Interface (numeric field – hexadecimal)
Set this field to the Interface number to use. The default is zero (0x0).

Alternate setting (numeric field – hexadecimal)
Any valid alternate setting value can be used here. The default is zero (0x0).

- 5.0. Example - Executing a Control Transfer Transaction Using SSTD 	Back to Top

This section explains how to create and execute a control transfer transaction using SSTD. Follow the steps to issue a Get Device Descriptor device request to the device.

- Run the SSTD program.

- If the  Select Host Controller dialog box appears, select the host controller to which the device under test is connected.

- Select the Config Setup Packets property sheet.

- Find the Get Descriptor command (9.4.3).

- Within that command, select Device from the Type drop-down box.

- Go back to the Main property sheet.

- Select the Get Descriptor request from the Select Transaction drop-down box.

- Click the Execute button to complete the Setup phase.

- Click Execute again to complete the Data Phase. If the device receives data, then it will appear in the Transmit/Receive buffer.

- Click Execute once more to complete the Status phase.

Please note that SSTD handles the Data Toggle and PID values automatically for the next transfer.

- 6.0. Troubleshooting					 				Back to Top

This section lists the possible SSTD error messages as well as the condition(s) that caused SSTD to display each error message. To report a defect in the SSTD program, please send e-mail to Daniel.S.Froelich@intel.com.

- “Could not get current directory string %Error Code%”
SSTD found a problem in getting the current directory string for the directory in which it tried to store the log file. The corresponding Win32* error code is displayed. It is possible that the directory does not exist; try a different directory name.

- “Could not get handle to history text”
SSTD was not able get a handle to the history buffer due to an internal resource conflict. Exit and restart SSTD. If this error message is still received, report this as an SSTD defect.

- “Could not start thread!”
SSTD was not able to start a new thread to carry out the selected transaction. The computer system might be overloaded; consider exiting other applications or running SSTD on a different system. It may be necessary to reboot the system.

- “Length of data (length in bytes) is not even or is empty”
User error; to remedy this, enter a data length that is an even number.

- “Could not open file %LogFileName% %ErrorCode%”
SSTD found a problem in opening the Logfile. The file name and corresponding Win32 error code is displayed. The file might already exist; try a different file name.

- “Bad PID entry”
User error; your PID selection is conflicting with other options. For example, if the transfer type selected is Isochronous, the selected PID cannot be SETUP (only the IN or OUT PIDs are allowed for an Isochronous transfer type; please see section 4.1.1). To remedy this, enter a PID that is appropriate for the transfer type or modify the transfer type.

- “Could not allocate memory %Purpose%”
SSTD was not able to allocate memory for the specified purpose. The computer system might be overloaded; consider exiting other applications or running SSTD on a different system. It may be necessary to reboot the system.

- “Opening HKEY_LOCAL_MACHINE\ %RegistryKey%  value failed”
SSTD was not able to find the named registry key in the Windows registry. A cause might be that the registry key does not exist because the registry is corrupted.

- “Could not acquire handle to HC”
SSTD was unable to communicate with the USB 2.0 host controller. Be sure that the host controller is installed in the computer system and that the appropriate drivers are loaded.

- “No host controller found”
SSTD could not find a USB 2.0 host controller in the computer system. Install at least one USB 2.0 host controller to use SSTD successfully.

- “No logfile entry %ErrorCode%”
User error; SSTD found that you tried to save history to a Logfile without specifying a file name. The corresponding Win32 error code appears. To remedy this, specify an appropriate file name.

- “Transaction error”
User error; SSTD found an error while attempting to process a user-specified transaction. Ensure that the specified transaction is legal. If the transaction is legal, report this as an SSTD defect.

- “Babble detected”
SSTD found babble on the USB bus. To remedy, unplug the device under test from the USB bus and then plug it back in.

- Back to Top

Information in this document is provided in connection with Intel® products. No license, express or implied, by estoppel or otherwise, to any intellectual property rights is granted by this document. Except as provided in Intel's Terms and Conditions of Sale for such products, Intel assumes no liability whatsoever, and Intel disclaims any express or implied warranty, relating to sale and/or use of Intel® products including liability or warranties relating to fitness for a particular purpose, merchantability, or infringement of any patent, copyright or other intellectual property right. Intel products are not intended for use in medical, life saving, or life sustaining applications. 
Intel may make changes to specifications and product descriptions at any time, without notice.

Copyright © 2000, Intel Corporation. All rights reserved. *Other product and corporate names may be trademarks of other companies and are used only for explanation and to the owners' benefit, without intent to infringe.