A Brief Introduction to RTL8192C driver Power Saving

1.) RTL8192C consists of two power modes:

A). Minimum Power Saving mode.

In this PS mode, RTL8192C wakes up every beacon interval. Figure 1 shows the RTL8192C power consumption under Minimum Power Saving mode.

Figure 1. Minimum Power Saving mode (Associated idle)

B). Maximum Power Saving mode:

When driver be set to maximum power saving mode, which means RTL8192C wake up to receive beacon when DTIM count of beacon is zero.

If DTIM period of beacon is 3 RTL8192C would wake up every 3 bacon interval. Figure 2 illustrates RTL8192C power consumption under Maximum Power Saving mode:

Figure 2. Maximum Power Saving mode (Associated idle)

2.) The following document will describe the way to let this driver to enable the power saving functionality.

===========================================================

A-1.) Power Saving Mode – by modify Makefile

===========================================================

You can easy set the power saving to Min-PS (rtw_power_mgnt=1) by modify the CONFIG_POWER_SAVING to y, at the line 20 of Makefile file.

Ex:

CONFIG_POWER_SAVING 	= 	y

===========================================================

A-2.) Power Saving Mode – by change load-time parameters (detailed setting instructions)

===========================================================

(1) In order to enter PS Mode, you need to add the parameter of "rtw_power_mgnt=1" when executing "insmod 8192cu.ko" :

$>insmod 8192cu.ko rtw_power_mgnt=1

or

$>insmod 8192cu.ko rtw_power_mgnt=2

(2)Parameter Notes:

rtw_power_mgnt=1 ;//enable PS, MIN_PS Mode

rtw_power_mgnt=2 ;//enable PS, MAX_PS Mode

===========================================================

B-1.) USB Autosuspend - by modify Makefile

===========================================================

You can turn on USB autosuspend function by modify the CONFIG_USB_AUTOSUSPEND to y, at the line 21 of Makefile file.

Ex:

CONFIG_USB_AUTOSUSPEND 	= 	y

===========================================================

B-1.) USB Autosuspend - by change load-time parameters

===========================================================

(1) In PS mode, you can turn on or turn off the USB autosuspend functionality by adding the following parameter.

$>insmod 8192cu.ko rtw_power_mgnt=1 enusbss = 1

or

$>insmod 8192cu.ko rtw_power_mgnt=1 enusbss = 0

or

$>insmod 8192cu.ko rtw_power_mgnt=2 enusbss = 1

or

$>insmod 8192cu.ko rtw_power_mgnt=2 enusbss = 0

(2)Parameter Notes:

enusbss = 1; // enable USB autosuspend

enusbss = 0; // default, disable USB autosuspend

3.) Power consumption example

| Scenario (unit: mA) @5V | RTL8188CUS |
| --- | --- |
| Associated Idle | 85.6 |
| Unassociated Idle | 80.2 |
| Associated Idle (enable power save) - LPS | 34.5 |
| Unassociated Idle (enable power save) - IPS | 18.9 |
| Unassociated Idle (enable selective suspend) | 0.72 |